Atari ST Machine Specific Programming In Assembly

Chapter 3: Assembly and Addressing Modes

On April 1st, one year in the late 1970's, the

announcement of a revolutionary semiconductor device, the

write only memory (WOM), provided engineers the laugh of the

day.  Imagine my mirth when I discovered that Motorola was

apparently manufacturing a microprocessor that is

"compatible" with that device--a read only processor (ROP).

Imagine my chagrin when I realized that I had purchased a

computer designed around that microprocessor.

In the early stages of my ST education, the dilemma I

faced was this: the ST's operating system requires position

independent programs (My definition of a position

independent program is this: it is a program for which the

validity of execution results does not depend on the

location of the program and its data in ram.); according to

the references, this attribute is bestowed on programs which

use the MC68000 pc-relative addressing mode to access label

operands; however, this addressing mode cannot be used when

the operand is a destination.  How, then, does one store

data in program variables during program execution?

The ST Relocator

Well, of course, I soon learned, via page 79 of the

AssemPro manual, that the ST has a built in program

relocator that nullifies the MC68000's pc-relative

addressing mode limitation, by producing position

independent code for programs that are relocatable (My

definition of a relocatable program is this: it is a program

to which the attribute position independence may or may not

initially be applicable.  Regardless, however, the relocator

will position the program in ram in a manner which

guarantees that the validity of execution results does not

depend on its location); and, fortunately, AssemPro provides

relocatable code when a source file is assembled with the

Relocatable assembly option checked.  For a time, therefore,

the dilemma seemed moot.

But the process is not as automatic as it seems.  The

AssemPro manual attempts to provide an example, on pages 79

and 80, to indicate a type of expression which the relocator

would find unpalatable.  I have tried to deduced Mr.

Schulz's intent, because it is not clearly stated.

Furthermore, the example does not seem to contain enough

information to render it informative.  I don't think that he

intended to indicate that files containing these types of

expressions will not be assembled, because I have assembled

my own example.  The source file is program 6a.

I have included various statements which use the

declared labels in this program so that you can see what

works and what doesn't.  Statement #5 is the only one that

causes a run time error.  And the reason is not a mystery,

as you can see in the disassembly listing shown in figure

3.2.  The statement assembles into an instruction that

attempts to store the contents of address -4 in register D0.

Program 6a. My source file for the discussion on pages 79

and 80 of the AssemPro manual.

; Program Name: PRG_2AR.S

;      Version: 1.002

; Assembly Instructions:

;    Assemble in AssemPro Relocatable mode and save the assembled program

; with a PRG extension.

; Program Function:

;    Serves as a source file for the discussion on pages 79 and 80 of the

; AssemPro manual.

move.l    up_1, d0

move.l    up_2, d0

move.l    #-4, d0        ; The statement below will resemble this one

; after assembly.  Both are relocatable.

move.l    #up_1-up_2,d0  ; This is Mr. Schulz's example.

move.l    up_1-up_2, d0  ; I think he meant to use this type of example,

data                     ; because this one does cause a problem.

pointer:   dc.l  up_1

up_1:      dc.l  5

up_2:      dc.l  3

end

This program was assembled in AssemPro's Relocatable

mode, with the Listing option checked.  The listing was

directed to the file shown in figure 3.1.  As you shall see

in figure 3.2, a disassembled listing of program 6a, the

expression #up_1-up_2 is evaluated to -4 during assembly.

This fact is also evident in the listing of figure 3.1,

where you are able to see that the object code on line 6 is

actually that for the instruction: move.l #-4, d0, identical

to the statement on line 5.  This is the instruction given

as an example of one that is not relocatable in the AssemPro

manual.  But I found that this instruction does not cause

any problems.  In fact, I included the statement on line 5

of the program to illustrate that it works whether it is

introduced specifically into the program or it is assembled

into that format by the assembler.  It is the statement on

line 19 which causes a run time error.

Figure 3.1. Assembly listing for program 6a.

Address Objectcode   Line    Sourcetext Assembly listing for PRG_2AR.S

000000 :                 1   ; Program Name: PRG_2AR.S

000000 :                 2   ;      Version: 1.002

000000 :                 3

000000 :                 4   ; Assembly Instructions:

000000 :                 5

000000 :                 6   ;    Assemble in AssemPro Relocatable mode and save the assembled program

000000 :                 7   ; with a PRG extension.

000000 :                 8

000000 :                 9   ; Program Function:

000000 :                10

000000 :                11   ;    Serves as a source file for the discussion on pages 79 and 80 of the

000000 :                12   ; AssemPro manual.

000000 :                13

000000 :203900000022    14   move.l    up_1, d0

000006 :203900000026    15   move.l    up_2, d0

00000C :203CFFFFFFFC    16   move.l    #-4, d0        ; The statement below will resemble this one

000012 :                17                            ; after assembly.  Both are relocatable.

000012 :203CFFFFFFFC    18   move.l    #up_1-up_2,d0  ; This is Mr. Schulz's example.

000018 :2039FFFFFFFC    19   move.l    up_1-up_2, d0  ; I think he meant to use this type of example,

00001E :                20   data                     ; because this one does cause a problem.

00001E :00000022        21  pointer:   dc.l  up_1

000022 :00000005        22  up_1:      dc.l  5

000026 :00000003        23  up_2:      dc.l  3

00002A :                24   end

The file shown in figure 3.2 was produced by clicking

on the Disassembling option under the Debugger menu, then by

typing $5D7B4 as the from address and $5D7DE as the to

address.  When disassembling a program with this AssemPro

function, you must always specify a to address that is

beyond the last address you want to appear in the listing;

furthermore, the address you specify must be beyond the

address of the last item of data you wish to appear in the

listing.  For example, the last address shown in the listing

of figure 3.2 is $05D7DA.  The data located at that address

is a longword (4 bytes) in length, therefore, the to address

specified was $05D7DA + 4 = $05D7DE.

Figure 3.2. Disassembly listing for program 6a.

Disassembled listing of PRG_2AR.PRG

05D7B4 20390005D7D6  MOVE.L $5D7D6,D0

05D7BA 20390005D7DA  MOVE.L $5D7DA,D0

05D7C0 203CFFFFFFFC  MOVE.L #-4,D0

05D7C6 203CFFFFFFFC  MOVE.L #-4,D0

05D7CC 2039FFFFFFFC  MOVE.L -4,D0

05D7D2 0005D7D6      ORI.B #-$2A,D5

05D7D6 00000005      ORI.B #5,D0

05D7DA 00000003      ORI.B #3,D0

The disassembly listing clearly shows that an error

will occur when an attempt is made to execute the fifth

statement because that statement has been assembled into one

which attempts to load the contents of a negative address

into D0.  See figure 3.3.  The -4 produced in this statement

(as is the -4 in the fourth statement) is the difference

between the addresses of labels up_1 and up_2 during

assembly.

Figure 3.3. Attempting to execute the fifth statement of

figure 3.2 in the AssemPro debugger will cause an alert box

similar to this one to appear.

If the intent of a programmer is to store the

difference between the contents of the run time addresses of

labels up_1 and up_2 in D0, then, clearly, that does not

happen.  I believe that this is the point which Mr. Schulz

intended to make with his example.  Even if this was not his

intent, the example still illustrates that assembly using

AssemPro's Relocatable mode does not guarantee a position

independent program, unless the elements of each statement

in the program are congruous with relocatability.  As we

have seen, it is possible to load and execute a position

dependent program which has been assembled in the

Relocatable mode.

A Few More Comments About the AssemPro Manual

The paragraph concerning the advantage of a relocatable

program on page 80 has not been correctly written.  Instead

of "The advantage of a relocatable program...", the

paragraph should merely indicate that labels can be used as

destination operands in a program that is assembled in

AssemPro's Relocatable mode.  Nothing else that is implied

by that paragraph can be confirmed.  That the use of labels

in destination operands is advantageous is a subjective

opinion at best.  In fact, I shall show that the requisite

memory of a program assembled in AssemPro's Relocatable mode

can never be as low as if it were assembled in the PC-

relative mode; and, furthermore, the loading times required

for programs assembled in the Relocatable mode are much

greater than they are for those assembled in the PC-relative

mode.

The following characteristics are the major differences

between the three AssemPro modes.  Some of these

characteristics are not evident during the assembly process.

1. When a program is assembled in PC-relative mode the

assembler automatically attaches a (PC) suffix to

labels used as source operands.  If the program were

assembled in the Absolute mode, the programmer would

have to manually attach the (pc) suffix to each

label.  During assembly, the assembler will issue an

error report, declaring that the effective address

is not acceptable, for each label used as a

destination operand.

2. In a program assembled in the Relocatable mode,

labels are accepted as legal destination operands.

If used, the (pc) suffix must be inserted where

desired by the programmer.  As in item 1, above, the

assembler will flag instructions which contain a

label(pc) in a destination operand.

3. Programs assembled in the Absolute or PC-relative

modes inherently require less memory and loading

time than do programs assembled in the Relocatable

mode.

Concept Definitions

The concepts with which we must be concerned during

this discussion involve assembly modes, run-time memory

addresses and assembly-time addresses.  During program

execution, addresses are referenced as data locations or as

locations which must be forced into the program counter in

order to effect a transfer of execution flow.  I will now

define the necessary concepts and the conditions under which

I will apply the terminology connected with each.  My

comments concerning things to which the word absolute

applies will be brief because our interests are in things to

which the words program counter relative and relocatable

apply.  Remember that PC-relative assembly is simply

Absolute assembly with (PC) automatically inserted for

convenience.  But remember also that there may be times when

absolute assembly would be preferable to PC-relative

assembly because of the 16-bit branch limitation associated

with pc-relative addressing.  When that limitation can't be

tolerated, automatic insertion of the (PC) suffix becomes an

inconvenience.

Concept One

The AssemPro assembler has three assembly options.

These are PC-relative, Relocatable and Absolute.  For our

convenience only, I will use the name of the mode in which a

program was assembled as an adjective to describe a program

assembled in that particular mode.  For example, I will say

things such as "this is a PC-relative program; that is a

Relocatable program.

In addition to the AssemPro modes, I add one of my own,

the Combination (Combo) mode.  A Combo program will be one

that is assembled in AssemPro's Relocatable mode, but one in

which the location of labels used as source operands in

instructions are specified using the MC68000 Program Counter

With Displacement addressing mode.  The Combo mode produces

programs that take advantage of the faster execution speed

and smaller memory requirements of the program counter with

displacement addressing mode, and the mode also permits

labels to be used in destination operands.

Concept Two

The MC68000 microprocessor has among its addressing

modes one which is called program counter with displacement,

one which is called program counter with index, one which is

called absolute short and one which is called absolute long.

We will be interested in the absolute short and absolute

long addressing modes when we wish to reference operating

system variables while the processor is in its supervisor

mode.  For the discussion in this chapter, we are primarily

interested in the program counter with displacement mode.

All of the references that I have seen replace this rather

cumbersome phrase with pc-relative, and refer to the

addressing mode as pc-relative addressing.  Now that I have

pointed out the difference between PC-relative assembly and

pc-relative addressing, we should be able to use this common

notation with no problem.

The Rub

Before I go further, because I am going to say what

needs to be said about the MC68000, I must clarify my

position.  I consider Motorola microprocessors to be the

finest available.  And I speak from experience with their

devices and Intel's.  Yet, were it not for a serious flaw

(one of two) in the design of the MC68000 (My opinion, of

course, but the only viewpoint from which a sane discussion

of the processor's limitations can proceed.), this

discussion would not be taking place.

However else are we to view the obstacle to storing

data that is inherent in the chip's design?  Are we to

believe that someone would deliberately incorporate an

addressing scheme which attempts to thwart data storage in

the design of a computer's processor?  That is the snow job

that most of the references parrot.  No, the extra coding

that is necessary to overcome the limitation of the

MC68000's pc-relative addressing mode makes sense, and is

justified, only when that limitation is viewed as a design

error.

The flaw, of which I now speak, is the microprocessor's

inability to accept pc-relative destination operands.

Ostensibly, according to the references, Motorola chose this

method to protect us from mad dog, crazy-running-amok, self-

modifying code producing programmers.  I shall prove to you

that the design of the pc-relative addressing mode does not

prevent the writing of data to destination operands; thus, I

must conclude that, if the limitation was designed

deliberately, it is a failure; therefore it is useless and

only hinders programming effort.

On the other hand, if the limitation was an error in

design, I must conclude that it was not deliberate, and,

being otherwise satisfied with the microprocessor, I choose

to accept the task of designing my programs to work around

the flaw.  That is this chapter's raison d'tre: designing

PC-relative programs that work in spite of the flaw.  The

extra effort that must be exerted is justified as

corrections to an honest mistake in design.  Furthermore, I

will try to show you that there exists a viewpoint from

which we can consider the flaw to be an incentive to

inspirational algorithmic development, the result of which

shall be algorithms that are faster and consume less memory

than they might have were we not forced to use the

corrections.

PC-Relative Addressing: The Microprocessor Option

I have decided that the most coherent explanation of

MC68000 pc-relative addressing is contained in the Kelly-

Bootle book.  See pages 59-70, 91-92, 112-115  and 186-196.

But, of course, Mr. Kelly-Bootle's explanation is not Atari

St specific; he did not intend that it be.  My explanation

will fill in the blanks, but I do advise you to read Mr.

Kelly-Bootle's pc-relative material, and, if you've the

time, the other sections of chapters 1 through 7 you find

pertinent.  I am especially fond of the quotation at the

beginning of his introduction.

The importance attached to pc-relative addressing on

page 61 of the Kelly-Bootle book do not apply to the ST

because of its built in relocator.  The assembly directive

RORG does not apply either, because AssemPro replaces that

directive with the PC-relative assembly option under the

Assembler menu.  The importance of the LEA and PEA

instructions in PC-relative programs is amply documented in

this book, but there is a compound error on page 113.  The

first part of the error is semantical.  MOVEA.Z #<label>, A0

cannot be both equivalent and faster than LEA <label>, A0;

if you don't see that, look up the definition of

equivalence.

The second part of the error is factual.  These

instructions execute at identical speeds according to the

Instruction Execution Times tables in the Motorola M68000

Programmer's Reference Manual.  Furthermore, after I have

covered sufficient introductory material, I will present a

program which provides data which supports the Motorola

manual.  In fact, if you think about it, why should the

times be different.  Both instructions accomplish the same

task.

PC-Relative Mode: The Assembler Option

Only programs which contain no destination operands that

are labels can be assembled with the assembler's PC-relative

mode asserted.  This means that you cannot write

instructions that store data in program variables by

referencing the labels of dc and bss declarations.  Instead,

you must store data in those locations via indirect

reference.  You do that by storing the location of the label

in an address register, then by using one of the MC68000's

Address Register Indirect addressing modes in the

destination operand portion of a data storing instruction.

This is the extra effort that must be expended to

nullify the flaw in the MC68000's Program Counter addressing

modes.  However, this effort does not always represent an

expense in speed and requisite memory.  Actually, expense is

incurred only when a single reference to a label is required

in a program, or when heavy register usage does not permit a

register to be assigned to a variable during a significant

portion of the program in which the variable must be

referenced.

When a program is assembled in the PC-relative mode,

the assembler forces the addressing mode of every label used

in an instruction operand into the MC68000 Program Counter

With Displacement addressing mode.  Of course, an error will

be reported for every instruction which contains a label in

the destination operand.  While it is true that programs

assembled in this mode are position independent, we are not

as interested in this attribute of PC-relative programs as

we are interested in their faster execution speed and lesser

requisite memory, because we can obtain position

independence simply by using the Relocatable mode.

Relocatable Mode: The Assembler Option

As we have seen, it is possible to construct statements

which the ST relocator cannot handle properly, even though

the assembler will not sense an error.  I avoid such

problems because I do not include mathematical expressions

in my instruction operands.  I have enough problems with my

programs without trying to outguess the relocator and the

assembler.

It may not be readily apparent that AssemPro's

Relocatable mode does not preclude the use of the MC68000's

program counter with displacement addressing mode.  I have

decided to discuss such usage as though it were another

assembly mode.  I do this only because Mr. Schulz stressed

the automatic inclusion of (PC) for labels used as source

operands when assembly takes place in his PC-relative mode,

and I want to stress the advantage of using the processor's

pc-relative mode in programs that are to be assembled in

AssemPro's Relocatable mode, even though (pc) must be

manually typed after ever label in a source operand.

Combination Mode: The Option Mr. Schulz Forgot

If AssemPro had been provided with an assembly mode

that inserted the (pc) for labels used as source operands,

but which simply ignored destination operands, then

assembled in Relocatable format, we would not have to type

(pc) behind each label in our source operands to take

advantage of the pc-relative addressing mode in Relocatable

programs.  As you shall see, by using the (pc) suffix

judiciously, we shall be able to reduce both execution time

and requisite memory.

Comparing Assembler Modes

The introduction of the program 6a provides me with the

opportunity to begin a comparative analysis of the

consequences of each Assembler mode.  You have already seen

three forms of a program assembled in the Relocatable mode.

I would like you to obtain one piece of information about

PRG_2AR.PRG from the desktop.  By clicking one on the

program's icon, then by selecting Show Info under the File

menu, you will be able to observe and record the disk space

occupied by the program.  It is 78 bytes.

You can obtain a PC-relative version of the program,

simply by loading PRG_2AR.S into the editor, clicking on

AssemPro's PC-relative Assembly option and assembling.

Change the file name to PRG_2AP and save the assembled

program to disk with a PRG extension.  From the desktop,

obtain the disk space occupied by the program (PRG_2AP.PRG).

It is 64 bytes.

Program 6b is a listing of PRG_2AR.S transformed into a

program that uses pc-relative addressing for the labels in

the source operands.  You should assemble this program in

the Relocatable mode, change the file name to PRG_2AC and

save the assembled code to disk as PRG_2AC.PRG.  From the

desktop, obtain the disk space occupied by the program.  It

is 72 bytes.

Program 6b. PRG_2AR.S altered to produce PRG_2AC.S, a Combo

program.

; Program Name: PRG_2AC.S

;      Version: 1.002

; Assembly Instructions:

;    Assemble in AssemPro Relocatable mode and save the assembled program

; with a PRG extension.

; Program Function:

;    Serves as a source file for the discussion on pages 79 and 80 of the

; AssemPro manual.

move.l    up_1(pc), d0

move.l    up_2(pc), d0

move.l    #-4, d0

move.l    #up_1-up_2,d0  ; (pc) can't be used with the labels here.

move.l    up_1-up_2, d0  ; (pc) can't be used with the labels here.

; NOTE: If the (pc) suffix is attached to both labels in the above

;       statement, the assembler will flag the first as an error.  If the

;       (pc) suffix is attached only to the second label, the instruction

;       will be assembled into a statement that moves a portion of the

;       the preceding statement into register D0.

data

pointer:   dc.l  up_1

up_1:      dc.l  5

up_2:      dc.l  3

end

The disassemby listing of PRG_2AR.S assembled in the

PC-relative mode is shown in figure 3.5.  There you can see

one of the reasons that PRG_2AP.PRG has a lower requisite

memory than PRG_2AR.PRG.  The first, second and fifth lines

of the disassembly listing show that the labels in these

instructions have been cast in the pc-relative addressing

mode by the assembler, therefore, these source operands

require only one word of extension, whereas those

instructions in PRG_2AR.PRG require two words of extension.

Six bytes of requisite memory are saved via the use of

the pc-relative addressing mode, two for each of the three

instructions.  The additional eight byte differential

between the requisite memory for PRG_2AR.PRG and PRG_2AP.PRG

is due to the information which must be stored for the

relocator in PRG_2AR.PRG, as discussed on page 80 of the

AssemPro manual.

Figure 3.5. Disassembly listing for program 6a assembled in

the PC-relative mode.

Disassembly listing of PRG_2AP.PRG

06D5DA 203A001A      MOVE.L $6D5F6(PC),D0

06D5DE 203A001A      MOVE.L $6D5FA(PC),D0

06D5E2 203CFFFFFFFC  MOVE.L #-4,D0

06D5E8 203CFFFFFFFC  MOVE.L #-4,D0

06D5EE 203AFFE6      MOVE.L $6D5D6(PC),D0

NOTE: Although the above statement has been assembled

without an error indication, the effective address

is not within the program's boundaries.

06D5F2 0000001C      ORI.B #$1C,D0

06D5F6 00000005      ORI.B #5,D0

06D5FA 00000003      ORI.B #3,D0

The disassembly listing for PRG_2AC.PRG, shown in

figure 3.6, illustrates that the extension word savings can

be obtained when the pc-relative addressing mode is applied

to appropriate instructions in programs that are assembled

in the Relocatable mode.  In PRG_2AC.PRG, the pc-relative

mode can be applied to the first and second instructions.

This yields a four-byte savings.  An additional 2 bytes are

saved because less information must be stored for the

relocator in this program.

Figure 3.6. Disassembly listing for program 6b.

Disassembly listing of PRG_2AC.PRG

06D4F6 203A001C      MOVE.L $6D514(PC),D0

06D4FA 203A001C      MOVE.L $6D518(PC),D0

06D4FE 203CFFFFFFFC  MOVE.L #-4,D0

06D504 203CFFFFFFFC  MOVE.L #-4,D0

06D50A 2039FFFFFFFC  MOVE.L -4,D0

06D510 0006D514      ORI.B #$14,D6

06D514 00000005      ORI.B #5,D0

06D518 00000003      ORI.B #3,D0

These somewhat trivial comparisons have been introduced

in order to prepare you for the more complicated comparisons

to follow.  Besides the requisite memory differentials

between the three programs, by referring to the disassembly

listings, you should notice that the correct run-time

address of label up_1 is stored at the label pointer when

the example program is assembled in the Relocatable or Combo

modes, but when it is assembled in the PC-relative mode, the

value stored at pointer is the assembly-time location of the

label up_1.  See figure 3.7.

Figure 3.7. Assembly listing for program 6a assembled in the

PC-relative mode.  Compare the assembly-time address of up_1

to the run-time address stored at pointer in figure 3.5 and

notice that they are the same.  That is not what was

desired.

Address Objectcode   Line    Sourcetext PRG_2AR.S assembled in PC-relative mode

000000 :                1   ; Program Name: PRG_2AR.S

000000 :                2   ;      Version: 1.002

000000 :                3

000000 :                4   ; Assembly Instructions:

000000 :                5

000000 :                6   ;    Assemble in AssemPro Relocatable mode and save the assembled program

000000 :                7   ; with a PRG extension.

000000 :                8

000000 :                9   ; Program Function:

000000 :               10

000000 :               11   ;    Serves as a source file for the discussion on pages 79 and 80 of the

000000 :               12   ; AssemPro manual.

000000 :               13

000000 :203A001A       14   move.l    up_1, d0

000004 :203A001A       15   move.l    up_2, d0

000008 :203CFFFFFFFC   16   move.l    #-4, d0        ; The statement below will resemble this one

00000E :               17                            ; after assembly.  Both are relocatable.

00000E :203CFFFFFFFC   18   move.l    #up_1-up_2,d0  ; This is Mr. Schulz's example.

000014 :203AFFE6       19   move.l    up_1-up_2, d0  ; I think he meant to use this type of example,

000018 :               20   data                     ; because this one does cause a problem.

000018 :0000001C       21  pointer:   dc.l  up_1

00001C :00000005       22  up_1:      dc.l  5

000020 :00000003       23  up_2:      dc.l  3

000024 :               24   end

In addition to the incompatibility between the

declarations and the PC-relative mode just discussed,

certain move instructions are not compatible with assembly

in the PC-relative mode.  Programs 7a-7d are sources

prepared for assembly in each of the four possible assembly

modes.  The name of each source listing indicates the mode

of assembly.  The comments in each program convey assembly

and run-time expectations and the suitability or

unsuitability of pertinent instructions relative to the mode

of assembly.  Program 8 is a final example that stresses the

differences between instructions which are compatible with

PC-relative assembly and those which aren't.  This program

can be assembled in Relocatable or PC-relative mode.  But

when it is assembled in PC-relative mode, the code for

portions of it is not what we want.

Program 7a. ABSOLUTE.S

; Program Name: Absolute.s

;      Version: 1.001

; Assembly Instructions:

;      Assemble in Absolute mode.

; Execution Instructions:

;      Go to the debugger and single step through the first 3 instructions.

; When you attempt to single step the 4th instruction a BUS ERROR will occur.

; Press the Return key when the alert box appears.  Then move the mouse arrow

; to the 5th instruction in the debugger output field and, while holding the

; Control key pressed, click on the left mouse button to move the program

; counter cursor to the 5th instruction.  Finally, single step the 5th

; instruction.

; Record the values that are stored in each of the four registers D0, D1,

; A0 and A1.

lea       label(pc), a0        ; Label's run time address is moved into A0.

move.l    label(pc), d0        ; Label's contents are moved into D0.

movea.l   #label, a1           ; Label's assembly time add is moved into A1.

; NOTE:  The MC68000 does not permit the (PC) suffix to be attached to a

;        label that is preceded by a # sign.

move.l    #$0, label

; Result of executing the above instruction is a BUS ERROR because label's

; address is its assembly time value, and that value is an address that can

; only be accessed while the processor is in supervisor mode.  You can prove

; that this is true by clicking on the status register S box in the debugger

; screen before executing the instruction again.  After that, I suggest that

; you reset the computer because storing $0 at that address may not be a

; healthy thing to do.

; NOTE:   Manually move the program counter cursor to the next instruction.

move.l    label(pc), d1        ; Label's contents are moved into D1.

label:     dc.l $FFFFFFFF       ; Label's contents initialized to -1.

end

Program 7b. COMBO.S

; Program Name: Combo.s

;      Version: 1.001

; Assembly Instructions:

;      Assemble in Relocatable mode.

; Execution Instructions:

;      Go to the debugger and click on the relocate button.  Single step

; through the 5 instructions.  Record the values that are stored in each

; of the four registers D0, D1, A0 and A1.

lea       label(pc), a0        ; Label's run time address is moved into A0.

move.l    label(pc), d0        ; Label's contents are moved into D0.

movea.l   #label, a1           ; Label's run time address is moved into A1.

move.l    #$0, label           ; Label's contents are changed to $00000000.

; NOTE:  If you want to see the new contents of label after the above

;        instruction is executed, click twice on the disassembled button

;        located in the lower right hand corner of the debugger screen.

move.l    label(pc), d1        ; Label's contents are moved into D1.

label:     dc.l $FFFFFFFF       ; Label's contents initialized to -1.

end

Program 7c. RELATIVE.S.

; Program Name: Relative.s

;      Version: 1.001

; Assembly Instructions:

;      Attempt to assemble in PC-relative mode.  The assembler will not

; accept the 4th instruction because of the attempt to use a label as a

; destination operand.  Type a semicolon in front of the instruction in

; order to comment it out, then assemble the program.

; Execution Instructions:

;      Go to the debugger and note that the assembler has attached a (PC)

; suffix to the source operand "label" in the 1st, 2nd and 4th instructions.

; Single step through the 4 instructions.  Record the values that are stored

; in each of the four registers D0, D1, A0 and A1.

lea       label, a0            ; Label's run time address is moved into A0.

move.l    label, d0            ; Label's contents are moved into D0.

movea.l   #label, a1           ; Label's assembly time address is moved into A1.

; NOTE:  The assembler will not attach a (PC) suffix to the source operand

;        "label" in the above instruction.

move.l    #$0, label

; Assembler will not accept "label" as a destination operand because AssemPro

; attaches a (PC) suffix to every label which is not preceded by a # sign in

; a program that is assembled in the PC-relative mode.  The MC68000 does not

; permit the (PC) suffix attachment to labels that are destination operands.

move.l    label(pc), d1        ; Label's contents are moved into D1.

label:     dc.l $FFFFFFFF       ; Label's contents initialized to -1.

end

Program 7d. RELOCATE.S

; Program Name: Relocate.s

;      Version: 1.001

; Assembly Instructions:

;      Assemble in Relocatable mode.

; Execution Instructions:

;      Go to the debugger and click on the relocate button.  Single step

; through the 5 instructions.  Record the values that are stored in each

; of the four registers D0, D1, A0 and A1.

lea       label, a0            ; Label's run time address is moved into A0.

move.l    label, d0            ; Label's contents are moved into D0.

movea.l   #label, a1           ; Label's run time address is moved into A1.

move.l    #$0, label           ; Label's contents are changed to $00000000.

; NOTE:  If you want to see the new contents of label after the above

;        instruction is executed, click twice on the disassembled button

;        located in the lower right hand corner of the debugger screen.

move.l    label, d1            ; Label's contents are moved into D1.

label:     dc.l $FFFFFFFF       ; Label's contents initialized to -1.

end

Program 8. Instructions that are compatible with PC-relative

assembly and those that aren't.

; Program Name: PRG_2BR.S

; Assembly Instructions:

;    The algorithms in this program can be assembled in Relocatable or

; PC-relative mode.  But when they are assembled in PC-relative mode, the

; code is not always what we want.

; Experiment 1.

;    Shows that a pointer, declared in the data section, to a variable

; declared in the bss section will contain the correct address when

; assembly is in Relocatable mode; but when assembled in PC-relative mode,

; the pointer will contain the location at which the variable resided

; during the assembly process.

movea.l   _variable, a0       ; A pointer to a variable is loaded into

; an address register.

; End of Experiment 1.

; Experiment 2.

;    Illustrates that the instructions

;         move.l #label, -(sp)

;         move.l #label, An

; are not compatible with assembly in the PC-relative mode, and that

; the following instructions must be used instead.

;         pea    label

;         lea    label.

move.l    #label_1, -(sp)

move.l    #label_1, a0

move.l    #label_2, -(sp)

move.l    #label_2, a1

pea       label_1

lea       label_1, a0

pea       label_2

lea       label_2, a1

; End of Experiment 2.

data

label_1:   dc.l      1

_variable: dc.l variable       ; _variable is a pointer to variable.

bss

label_2:   ds.l      1

variable:  ds.l      1         ; During loading, we want the address of

; this variable to be stored in the

; location addressed by the pointer.

end

You should assemble this program in both the

Relocatable and PC-relative modes so that you can compare

the assembly and disassembly listings.  My assembly listings

appear in figures 3.8 and 3.9.  The disassembly listings are

shown in figures 3.10 and 3.11.  The assembly-time addresses

of the variables declared in the data and bss segments of

the program are evident in figures 3.8 and 3.9.  These

should be compared to the addresses that would be loaded

into the arrays and the stack at run-time, as shown in

disassembly listings.

Figure 3.8. Assembly listing for PRG_2BR.S assembled in the

PC-relative mode.

Address Objectcode  Line    Sourcetext PRG_2BR.S Assembled in PC-relative mode

000000 :                1   ; Program Name: PRG_2BR.S

000000 :                2

000000 :                3   ; Assembly Instructions:

000000 :                4

000000 :                5   ;    The algorithms in this program can be assembled in Relocatable or

000000 :                6   ; PC-relative mode.  But when they are assembled in PC-relative mode, the

000000 :                7   ; code is not always what we want.

000000 :                8

000000 :                9   ; Experiment 1.

000000 :               10

000000 :               11   ;    Shows that a pointer, declared in the data section, to a variable

000000 :               12   ; declared in the bss section will contain the correct address when

000000 :               13   ; assembly is in Relocatable mode; but when assembled in PC-relative mode,

000000 :               14   ; the pointer will contain the location at which the variable resided

000000 :               15   ; during the assembly process.

000000 :               16

000000 :207A002E       17   movea.l   _variable, a0       ; A pointer to a variable is loaded into

000004 :               18                                 ; an address register.

000004 :               19   ; End of Experiment 1.

000004 :               20

000004 :               21   ; Experiment 2.

000004 :               22

000004 :               23   ;    Illustrates that the instructions

000004 :               24

000004 :               25   ;         move.l #label, -(sp)

000004 :               26   ;         move.l #label, An

000004 :               27

000004 :               28   ; are not compatible with assembly in the PC-relative mode, and that

000004 :               29   ; the following instructions must be used instead.

000004 :               30

000004 :               31   ;         pea    label

000004 :               32   ;         lea    label.

000004 :               33

000004 :2F3C0000002C   34   move.l    #label_1, -(sp)

00000A :207C0000002C   35   move.l    #label_1, a0

000010 :2F3C00000034   36   move.l    #label_2, -(sp)

000016 :227C00000034   37   move.l    #label_2, a1

00001C :               38

00001C :487A000E       39   pea       label_1

000020 :41FA000A       40   lea       label_1, a0

000024 :487A000E       41   pea       label_2

000028 :43FA000A       42   lea       label_2, a1

00002C :               43

00002C :               44   ; End of Experiment 2.

00002C :               45

00002C :               46   data

00002C :00000001       47  label_1:   dc.l      1

000030 :00000038       48  _variable: dc.l variable       ; _variable is a pointer to variable.

000034 :               49   bss

000034 : ^     4       50  label_2:   ds.l      1

000038 : ^     4       51  variable:  ds.l      1         ; During loading, we want the address of

00003C :               52                                 ; this variable to be stored in the

00003C :               53                                 ; location addressed by the pointer.

00003C :               54   end

Figure 3.9. Assembly listing for PRG_2BR.S assembled in the

Relocatable mode.

Address Objectcode  Line    Sourcetext PRG_2BR.S Assembled in Relocatable mode

000000 :                1   ; Program Name: PRG_2BR.S

000000 :                2

000000 :                3   ; Assembly Instructions:

000000 :                4

000000 :                5   ;    The algorithms in this program can be assembled in Relocatable or

000000 :                6   ; PC-relative mode.  But when they are assembled in PC-relative mode, the

000000 :                7   ; code is not always what we want.

000000 :                8

000000 :                9   ; Experiment 1.

000000 :               10

000000 :               11   ;    Shows that a pointer, declared in the data section, to a variable

000000 :               12   ; declared in the bss section will contain the correct address when

000000 :               13   ; assembly is in Relocatable mode; but when assembled in PC-relative mode,

000000 :               14   ; the pointer will contain the location at which the variable resided

000000 :               15   ; during the assembly process.

000000 :               16

000000 :20790000003A   17   movea.l   _variable, a0       ; A pointer to a variable is loaded into

000006 :               18                                 ; an address register.

000006 :               19   ; End of Experiment 1.

000006 :               20

000006 :               21   ; Experiment 2.

000006 :               22

000006 :               23   ;    Illustrates that the instructions

000006 :               24

000006 :               25   ;         move.l #label, -(sp)

000006 :               26   ;         move.l #label, An

000006 :               27

000006 :               28   ; are not compatible with assembly in the PC-relative mode, and that

000006 :               29   ; the following instructions must be used instead.

000006 :               30

000006 :               31   ;         pea    label

000006 :               32   ;         lea    label.

000006 :               33

000006 :2F3C00000036   34   move.l    #label_1, -(sp)

00000C :207C00000036   35   move.l    #label_1, a0

000012 :2F3C0000003E   36   move.l    #label_2, -(sp)

000018 :227C0000003E   37   move.l    #label_2, a1

00001E :               38

00001E :487900000036   39   pea       label_1

000024 :41F900000036   40   lea       label_1, a0

00002A :48790000003E   41   pea       label_2

000030 :43F90000003E   42   lea       label_2, a1

000036 :               43

000036 :               44   ; End of Experiment 2.

000036 :               45

000036 :               46   data

000036 :00000001       47  label_1:   dc.l      1

00003A :00000042       48  _variable: dc.l variable       ; _variable is a pointer to variable.

00003E :               49   bss

00003E : ^     4       50  label_2:   ds.l      1

000042 : ^     4       51  variable:  ds.l      1         ; During loading, we want the address of

000046 :               52                                 ; this variable to be stored in the

000046 :               53                                 ; location addressed by the pointer.

000046 :               54   end

Figure 3.10. Disassembly listing for PRG_2BR.S assembled in

the PC-relative mode.

PRG_2BR.S assembled in PC-relative mode

05A1E6 207A002E      MOVEA.L $5A216(PC),A0

05A1EA 2F3C0000002C  MOVE.L #$2C,-(A7)

05A1F0 207C0000002C  MOVEA.L #$2C,A0

05A1F6 2F3C00000034  MOVE.L #$34,-(A7)

05A1FC 227C00000034  MOVEA.L #$34,A1

05A202 487A000E      PEA $5A212(PC)

05A206 41FA000A      LEA $5A212(PC),A0

05A20A 487A000E      PEA $5A21A(PC)

05A20E 43FA000A      LEA $5A21A(PC),A1

05A212 00000001      ORI.B #1,D0         ; location of label_1.

05A216 00000038      ORI.B #$38,D0       ; location of _variable.

05A21A 542E5300      ADDQ.B #2,$5300(A6) ; location of label_2.

05A21E 4620          NOT.B -(A0)         ; location of variable.

05A220 5259          ADDQ.W #1,(A1)+

Looking at the first instruction in figure 3.10, you

can see that the value $38, which is the contents of memory

location $05A216, will be loaded into register A0.  With

this instruction, the value we wanted to store in A0 is the

run-time address of variable.  Referring to figure 3.11, you

can see that this desire is fulfilled when the program is

assembled in the Relocatable mode.  This is a good time to

point out that, although the disassembler indicates the

actual address from which the value will be taken in the

first instruction of figure 3.10, the object code itself

references the location relatively.

The object code at address $05A1E6 can be divided into

the instruction word and the extension word as shown below.

Instruction word: 207A

Extension Word:   002E

The instruction word can be divided into its component bits

as shown below:

Component  1  2  3   4   5   6

00 10 000 001 111 010

The first component of the instruction word indicates a

movea operation; the second component indicates that the

operation is word size.  Remember, our instruction indicated

that the operation was to be longword size.  The operation

size is reduced automatically because the effective address

is now specified in the pc-relative addressing mode.  Even

so, the word size operand will be extended to a 32 bit

quantity before the operation is done, because the

destination is an address register.

The third component of the instruction word indicates

that the destination register is 0, and the fourth component

indicates that the destination is an address register.  The

fifth and sixth components indicate that effective address

of the source operand is specified by pc-relative

addressing.  The displacement to be used in calculating the

address of the source operand is specified in the extension

word; that value is $2E.

The actual address of the source operand is calculated

by adding $2E to the contents of the program counter when it

contains the address of the extension word.  That is,

$05A1E8 + $2E = $5A216.  The disassembler kindly shows us

the results of the computation in the disassembly listing.

Referring to the disassembly listing for the Relocatable

program, in figure 3.11, you can see that no such

computation is necessary because the actual address of the

source operand is included in the object code for the first

instruction.

Figure 3.10 also shows that the values pushed onto the

stack or loaded into the address registers  by the next four

instructions of the program will produce equally undesirable

results.  Then, the four instructions which follow those

four illustrate the use of lea and pea to load the correct

run-time addresses into address registers and the stack in a

PC-relative program.  For example, the first pea instruction

will push the address equal to the value in the extension

word, $E, plus the address of the extension word, $5A204,

which is $5A212, the address of label_1.

Figure 3.11. Disassembly listing for PRG_2BR.S assembled in

the Relocatable mode.

PRG_2BR.S assembled in Relocatable mode

05A1E6 20790005A220  MOVEA.L $5A220,A0

05A1EC 2F3C0005A21C  MOVE.L #$5A21C,-(A7)

05A1F2 207C0005A21C  MOVEA.L #$5A21C,A0

05A1F8 2F3C0005A224  MOVE.L #$5A224,-(A7)

05A1FE 227C0005A224  MOVEA.L #$5A224,A1

05A204 48790005A21C  PEA $5A21C

05A20A 41F90005A21C  LEA $5A21C,A0

05A210 48790005A224  PEA $5A224

05A216 43F90005A224  LEA $5A224,A1

05A21C 00000001      ORI.B #1,D0         ; location of label_1.

05A220 0005A228      ORI.B #$28,D5       ; location of _variable.

05A224 00000002      ORI.B #2,D0         ; location of label_2.

05A228 06060606      ADDI.B #6,D6        ; location of variable.

So far, I have concentrated on the requisite memory

advantage of pc-relative addressing, whether is is

accomplished via the PC-relative assembly mode or by forcing

the mode on labels which are source operands in programs

assembled in the Relocatable mode.  In chapter 6 I will

focus attention on the execution speed advantage of pc-

relative addressing and the loading speed advantage of

programs assembled in AssemPro's PC-relative mode.

Emphasizing Speed

Writing algorithms that execute rapidly requires

discipline and dedication to the task of learning which

instructions execute fastest and a somewhat fanatical desire

to seek out those less obvious instruction combinations

which execute faster than those which seem to flow from the

intellect much faster than their execution time.  Magazine

articles are probably the best reference material for coding

that concentrates of speed improvement.

An excellent example of such an article, which

discusses speed and traps (not the exception kind), is 68000

Tricks and Traps, by Mike Morton, published in Byte,

September, 1986, pages 163-172.  This is article is so

valuable, I suggest that you write to Byte for a reprint, or

try to obtain one from a library.  During your search for

the fastest algorithms, I caution you to believe only what

you prove with your own programs.  Some references advance

claims that seem to be untried and unproven.

For example, Mr. Morton advances the following

conclusion: multiplying a word by 4 is faster with

add.w dn,dn

add.w dn,dn

than                  asl.w #2, dn.

I will present a program which refutes this assertion in

chapter 6.  Also, in his section on Fast subroutine calls,

Mr. Morton seems to indicate that the LEA instruction always

references labels with pc-relative addressing; this is not

true for programs assembled in AssemPro's Relocatable mode,

unless the pc-relative addressing is forced with label(pc).

The first stage of increasing a program's execution

speed is getting it into ram as fast as possible.  Some ways

of doing this is by moving from a ram disk to main ram,

using utilities such as Switch/Back, produced by Alpha

Systems (I use this product.), and by reducing the loading

time.  This latter method will be discussed in chapter 6.

Before I can introduce the algorithm that I shall use to

measure a program's load and execute time, I must present

the introductory material which makes the algorithm

comprehensible.

Conclusion

In this chapter my intent was to show you the

difference between programs produced by four assembly modes.

In future chapters, the emphasis will be placed on programs

assembled in the PC-relative mode.  In addition, I have

distinguished pc-relative addressing from AssemPro's PC-

relative assembly mode.

The basics of pc-relative addressing is covered in the

references, most comprehensively in the Kelly-Bootle book.

But you will see more examples of its use in the programs to

be presented in future chapters of this book.  I have

assumed the task of explaining the significance of

AssemPro's PC-relative and Relocatable assembly modes, and I

have suggested that programs which explicitly use pc-

relative addressing and are assembled in the Relocatable

mode are best understood when their mode of assembly is

referenced as if it were a fourth option.